# Arduino Corridor Cleaner — Full Build Guide (Vacuum + Mop + Obstacle Avoidance + Clap Control + Zoning + Auto‑Return)

Scope: Small, course‑demo robot designed for school corridor cleaning. Selects vacuum on dry floor and mop on wet floor using metal‑contact wet sensor. Adds ultrasonic obstacle avoidance, clap‑based start/stop, basic zoning/coverage mapping, and battery auto‑return to dock via a tape guide (line follow).

## 1) System Overview

Core controller: Arduino Uno (ATmega328P)

Functional blocks - Mobility: 2 × DC gear motors + caster, driven by TB6612FNG (or L298N if already available). - Vacuum: 12 V mini centrifugal blower, MOSFET‑switched. - Mopping: Mop pad on a hinged arm (SG90 servo), 6–12 V mini diaphragm/peristaltic pump to wet mop (MOSFET‑switched) + small water tank. - Wet/Dry decision: Stainless metal contacts under front bumper; AC‑excited sensing to reduce corrosion. - Obstacle avoidance: HC‑SR04 ultrasonic on a small pan servo (SG90) for left‑center‑right scan. - Sound control: Electret mic module with comparator (LM393 board or MAX9814 mic amp). Detect double‑clap pattern. - Zoning / basic mapping: Wheel encoders + IMU (MPU‑6050) for dead‑reckoning; simple grid coverage (boustrophedon stripes) with corridor boundaries. - Auto‑return to dock: Low‑battery detection via voltage divider; follow a black tape guide using 2× TCRT5000 line sensors back to a docking station; spring contacts for charging.

Top‑level state machine

IDLE ↔ WAIT_FOR_CLAP
  │ (start)
  ▼
EXPLORE/CLEAN ──► AVOID_OBSTACLE ──► resume path
  │                    │
  ├── (wet) → CLEAN_WET (mop+pump)
  └── (dry) → CLEAN_DRY (vacuum)
  │
  └── (low battery) → RETURN_HOME (find tape) → DOCKING → CHARGING → IDLE

## 2) Bill of Materials (BOM)

### Control & Power

- 1× Arduino Uno (or Nano to save space)

- 1× Motor driver TB6612FNG (preferred) or L298N

- 1× Buck converter (LM2596) 12 V → 5 V for logic/servos

- 1× Battery pack: 2S Li‑ion (2×18650, 7.4 V nominal) or 3S (11.1 V) depending on blower; include a BMS board (2S/3S) rated ≥10 A

- 1× Charger module at dock: TP5100 (for 2S) or appropriate multi‑cell charger

- 1× Fuse (e.g., 10 A inline) + reverse‑polarity protection (Schottky diode or ideal diode board)

- 2× Power MOSFET logic‑level (e.g., IRLZ44N/IRLZ34N) + flyback diode for pump (1N5819/SS34) and blower (if brushed)

- 1× Power switch, wiring, terminal blocks

### Mobility & Sensing

- 2× 12 V DC gear motors with wheels (with built‑in encoders preferred), 1× caster

- 1× HC‑SR04 ultrasonic

- 2× SG90 servos (one for ultrasonic pan, one for mop arm)

- 2× TCRT5000 line sensors (for tape guide to dock)

- 1× MPU‑6050 IMU (I²C)

- 2× Hall sensors (A3144) + 2× magnets if motors lack encoders

- 1× Electret microphone sensor: LM393 sound sensor or MAX9814 mic amp module (better SNR)

- Metal wet contacts: 2× stainless screws/plates + mounting hardware; 10 kΩ and 100 kΩ resistors; 1× 1 µF cap; optional 74HC14 Schmitt trigger

### Cleaning Hardware

- 1× Mini centrifugal blower (12 V) + duct to suction inlet

- 1× Mini diaphragm or peristaltic pump (6–12 V) + silicone tubing

- 1× Water tank (100–250 ml) + valve/check‑valve

- 1× Mop pad (microfiber) + hinged arm/bracket

- 1× Debris bin with mesh filter

### Dock & Chassis

- Dock base with black tape path from corridor to dock

- 2× Spring contacts (pogo pins or copper pads) on robot and matching on dock

- 1× 12 V DC adapter for dock (current ≥ blower + pump + charge overhead as needed)

- Chassis plate, standoffs, screws, cable ties; splash guards

### Optional/UX

- 1× 0.96” OLED (I²C) for status

- LEDs/Buzzer for feedback

## 3) Power & Wiring Architecture

Suggested rails (example with 3S pack): - Battery (11.1–12.6 V) → - Buck to 5 V for Arduino + sensors + servos - Direct to blower (if 12 V) through MOSFET - Direct to pump (6–12 V) through MOSFET (use buck to 9 V if your pump is 6–9 V)

Grounds: All module grounds must share a common GND.

Battery sensing: Voltage divider to Arduino A0 (e.g., 100 kΩ top, 33 kΩ bottom for ≤ 16.8 V full‑scale). Add 0.1 µF to A0‑GND.

Flyback protection: Pump and any brushed motors need diodes across coils. The blower often contains a motor; add diode if brushed.

Typical pin map (adjust as you like): - Motors (TB6612): - AIN1 D7, AIN2 D8, PWMA D5 (ENA) - BIN1 D12, BIN2 D13, PWMB D6 (ENB) - STBY D4 - Encoders: - Left: D2 (INT0), D3 (INT1) - Right: A2, A3 (if simple single‑channel) - Ultrasonic (HC‑SR04): TRIG D9, ECHO D10 - Pan servo: D11 (Servo) - Mop arm servo: D3 (or any PWM pin; juggle with encoders as needed) - Sound sensor: A1 (analog) or D14/A0 if digital comparator output - Wet sensor contacts: Drive pin DA7 (A7 as digital), Sense pin A6 (analog) UNO analog‑only pins A6/A7 are input‑only; use D7.. if you need drive; see circuit below - Line sensors: A4, A5 (analog) - IMU (MPU‑6050): I²C on A4 (SDA), A5 (SCL) — if these are used for line sensors, move line sensors to A1/A2 - OLED: I²C (shared) - Blower MOSFET gate: DA2 (or any free digital) - Pump MOSFET gate: DA1 (or any free digital)

Reconcile pins based on your exact shields/modules. Ensure timer/PWM availability for motor speed control.

## 4) Metal‑Contact Wet Sensor (Corrosion‑Aware Design)

Goal: Decide wet vs dry at the leading edge of the robot.

Why AC excitation? DC across contacts causes electrolysis/corrosion. We’ll alternately drive and sense to pseudo‑AC excite.

Simple robust approach: - Contacts: Two stainless steel screws ~5–8 mm apart, 2–3 mm above floor, mounted on an insulating bracket. Add a thin sponge/guard to prevent shorting by metal debris. - Circuit: 1. Drive one contact with a 1 kHz square wave from Arduino (digital pin through 1 kΩ resistor). 2. Sense the other contact on an analog input through a 100 kΩ resistor + 1 µF to ground (forms an envelope detector). 3. Optional 74HC14 after a voltage divider for a clean digital edge. - Firmware: - Toggle the drive pin HIGH/LOW every 0.5 ms. - Sample the analog sense; high readings indicate conduction (wet). - Use a moving average and hysteresis thresholds (e.g., WET_ON > 250, WET_OFF < 200 on 10‑bit ADC) to avoid chatter.

Alternative super‑simple (more corrosion): Use internal pull‑up on sense pin; set drive pin LOW; when water bridges, the sense pin reads LOW. If you choose this, pulse the drive (duty < 10%) to reduce corrosion.

## 5) Obstacle Avoidance

- Mount HC‑SR04 on pan servo (±45°). Each cycle: look center, left, right and take median of 3 pings at each angle.

- If distance < D_STOP (e.g., 20 cm), stop → back up 10 cm → rotate toward open direction.

- Add a short blind‑time after a turn to avoid re‑triggering on the same obstacle.

## 6) Sound‑Triggered Control (Clap Toggle)

- Use MAX9814 module (preferred) to get a stable envelope; or LM393 sound sensor.

- Detect two claps within 500–800 ms windows:

  - Threshold by dynamic baseline (median filter) + fixed offset.

  - Debounce each clap (ignore events for 150 ms after a peak).

  - On valid double‑clap: toggle IDLE ↔ CLEAN.

## 7) Vacuum vs Mop Actuation Logic

- Dry (not wet): Enable blower, lift mop arm (servo to UP), pump OFF.

- Wet: Disable blower, lower mop arm (servo to DOWN), pulse the pump periodically (e.g., 200 ms ON every 2–3 s) to dampen pad but avoid puddles.

- Add a post‑wet drying pass: after leaving a wet patch, keep mop down for extra 20 cm then lift.

## 8) Zoning & Basic Mapping (Corridor‑Friendly)

Assumptions: Corridor ~ long rectangle. Demo uses dead‑reckoning (encoders + IMU yaw) and a simple grid.

- Grid: 10 cm cells; e.g., 20×5 grid stored as bytes (visited flags) → ~100 B.

- Coverage pattern: Boustrophedon (lawnmower): sweep forward along the corridor length; at end or obstacle, shift sideways by 1 strip (corridor width/n_stripes), then sweep back.

- Strip alignment: Use IMU yaw to maintain heading; re‑zero yaw after each turn. Encoders estimate distance.

- Zones: Divide the corridor into named segments (Z1, Z2, …) by distance from dock (e.g., every 3 m). Store current zone_id for UI/logs.

- Persistence: For a demo, map may be RAM‑only. Optionally store coverage percentage in EEPROM every minute.

This is not SLAM. It’s deterministic coverage using odometry suitable for a course demo.

## 9) Auto‑Return & Docking

Low‑battery detection: - Compute pack voltage from divider; compare to thresholds (e.g., 2S: LOW=7.2 V, CRITICAL=7.0 V). Add 100–200 ms RC filtering + software averaging.

Return strategy (simple & robust): - Find guide tape: On low battery, rotate in place while reading 2× TCRT5000; when both see black, align and line‑follow back to dock. - Docking: Slow approach (PWM ~30%), keep center to tape. When front bump/limit switch (optional) or charging voltage detected, stop motors → CHARGING state.

Dock electronics: - Dock has 12 V adapter → charger module (TP5100 for 2S) → spring contacts. - Robot’s pack connects through BMS to those contacts. Add LED indicators on dock for charging/full.

If you cannot lay tape in the corridor, add a cheap IR beacon at the dock and do rotational homing (but tape is far simpler for a demo).

## 10) Software Architecture

Recommended libraries: - AccelStepper (if you ever use steppers) / or simple PWM for DC - Servo.h - NewPing.h (or your own HC‑SR04 timing) - Wire.h + MPU‑6050 helper - Optional: EEPROM.h, small PID lib for heading hold

Core modules: - Motion: set speeds, turns, distance using encoders + yaw hold - Sensors: ultrasonic scan, wet sensor filter, line sensors, sound detect, battery voltage - Actuators: blower, pump, mop servo, pan servo - CoveragePlanner: boustrophedon strips across grid - FSM: handles states & transitions

State summary: - IDLE, WAIT_FOR_CLAP, CLEAN_DRY, CLEAN_WET, AVOID_OBSTACLE, RETURN_HOME, DOCKING, CHARGING

## 11) Key Circuits (Text Description)

A) Wet sensor (AC‑excited): - Arduino D7 → 1 kΩ → Contact A - Contact B → 100 kΩ → A0 (sense) - A0 → 1 µF → GND - Add 220 kΩ from A0→GND as a bleed if needed - Firmware toggles D7 at ~1 kHz

B) Battery monitor: - Pack + → 100 kΩ → A1 → 33 kΩ → GND, 0.1 µF A1→GND - Calibrate with multimeter

C) MOSFET driver (blower/pump): - Arduino D8 (gate) → 100 Ω → MOSFET G - 10 kΩ from G→GND - MOSFET D to motor −; motor + to battery + (or buck output) - MOSFET S to GND - Flyback diode across motor

D) Line sensors: - TCRT5000 analog outputs to A2/A3; power from 5 V; small trimmer sets threshold if using digital modules

E) Encoders: - If present, connect to D2/D3 (interrupts). If absent, add hall + magnets on wheel hubs.

## 12) Tuning Values (Starting Points)

- Ultrasonic thresholds: D_STOP=20 cm, D_TURN=30 cm

- Wet sensor ADC: WET_ON=250, WET_OFF=200 (10‑bit scale)

- Line sensor: black tape threshold ~ < 400 on 10‑bit ADC (depends on module/height)

- Clap detection: envelope threshold = baseline + 80 (tune), double‑clap window 600 ms

- Mop servo angles: UP = 30°, DOWN = 120° (depends on linkage)

- Pump pulse: 200 ms ON every 2–3 s while moving on wet

## 13) Pseudocode (Skeleton)

setup() {
  initPins(); initMotors(); initServos(); initIMU(); initADC();
  calibrateLine(); calibrateWet(); baselineSound();
  state = IDLE;
}

loop() {
  readAllSensors();
  updateWetFiltered();
  updateBattery();
  if (lowBattery() && state != RETURN_HOME && state != DOCKING && state != CHARGING) state = RETURN_HOME;

  switch(state) {
    case IDLE:
      stopAll(); blowerOff(); pumpOff(); mopUp();
      if (doubleClapDetected()) state = CLEAN_DRY; // will switch internally based on wet
      break;

    case CLEAN_DRY:
      blowerOn(); pumpOff(); mopUp();
      followCoverageStrip();
      if (wetDetected()) state = CLEAN_WET;
      if (obstacleAhead()) state = AVOID_OBSTACLE;
      break;

    case CLEAN_WET:
      blowerOff(); mopDown(); pumpPulse();
      followCoverageStrip();
      if (!wetDetected()) { postWetTimer(); if (timerDone()) mopUp(); state = CLEAN_DRY; }
      if (obstacleAhead()) state = AVOID_OBSTACLE;
      break;

    case AVOID_OBSTACLE:
      stopAll(); backUp(10); turnTowardOpen();
      state = (wetDetected() ? CLEAN_WET : CLEAN_DRY);
      break;

    case RETURN_HOME:
      searchForTape();
      if (tapeFound()) { lineFollowToDock(); state = DOCKING; }
      break;

    case DOCKING:
      approachSlow();
      if (chargingDetected()) { stopAll(); state = CHARGING; }
      else if (timeout) { // couldn't dock
        state = RETURN_HOME; }
      break;

    case CHARGING:
      if (packFull()) state = IDLE;
      break;
  }
}

Helper: wet detection (AC‑excited)

// called in timer ISR or fast loop
void driveWetPin() { digitalWrite(WET_DRIVE_PIN, !digitalRead(WET_DRIVE_PIN)); }

void updateWetFiltered() {
  int raw = analogRead(WET_SENSE_PIN);
  wetEMA = (wetEMA*7 + raw)/8; // simple low‑pass
  if (!wetState && wetEMA > WET_ON) wetState = true;
  if (wetState && wetEMA < WET_OFF) wetState = false;
}

Helper: double‑clap

void updateSound() {
  int v = analogRead(SOUND_PIN);
  baseline = 0.99*baseline + 0.01*v;
  if (v > (baseline + THRESH)) { if (millis()-lastClap>150) {claps++; lastClap=millis();} }
  if (claps==2 && (millis()-lastClap)<800) { toggleRequested=true; claps=0; }
  if ((millis()-lastClap)>800) claps=0;
}

Helper: coverage

void followCoverageStrip() {
  holdHeading(yawTarget);
  setSpeed(BASE_SPEED);
  if (distanceAlongStrip() > STRIP_LEN || endDetected()) {
    stopAll(); shiftSideways(STRIP_SPACING); yawTarget = normalize(yawTarget+180);
    resetDistance();
  }
}

## 14) Build Steps (Step‑by‑Step)

- Chassis & drivetrain: Mount motors+wheels, caster, battery bay, electronics plate. Ensure ground clearance for mop.

- Vacuum module: Fit blower and duct; add debris bin/filter; confirm airflow.

- Mop assembly: Hinge + servo horn; test UP/DOWN angles; mount pad; route tubing from pump and tank to pad.

- Sensors: Install ultrasonic on pan servo (front top), line sensors near floor (front underside), metal contacts (front edge), mic module (away from motors), IMU (center of mass).

- Power wiring: Battery → fuse/BMS → buck (5 V) → Arduino/servos/sensors; MOSFETs for blower/pump; common ground.

- Dock: Mount spring contacts, charger board, and lay black tape path. Verify polarity before first dock.

- Firmware bring‑up:

  - Blink test; read battery voltage and calibrate divisor.

  - Read IMU yaw; confirm stable heading.

  - Test encoders & straight‑line driving.

  - Test ultrasonic pings and pan scan.

  - Test wet sensor raw values: dry vs damp cloth; tune WET_ON/OFF.

  - Test mop servo limits; pump pulse.

  - Test blower MOSFET.

  - Test line sensors over tape; tune thresholds.

  - Test clap detection and toggle.

- Integrate FSM: Start with CLEAN_DRY only → add wet logic → add avoidance → add return/docking.

- Field trial: In a 2–3 m mock corridor, confirm coverage stripes, wet/dry switching, and docking.

## 15) Safety & Practical Notes

- Water + electronics: Use splash guards; keep 5 V electronics elevated. Conformal coat exposed PCBs near floor.

- Current draw: Blower can spike several amps—size MOSFETs, wiring, and fuse appropriately.

- Electrolysis: Prefer AC‑style excitation for wet sensor; rinse contacts after salty spills.

- Servos: Avoid stalling (set mechanical end‑stops). Provide dedicated 5 V rail with enough current (≥1 A burst per servo).

- Battery care: Never charge Li‑ion without a proper charger/BMS. Verify dock polarity.

- Noise: Clap control may falsely trigger in loud environments; consider long‑press button as secondary start/stop.

## 16) Minimal Parts Substitution Guide

- No encoders? Use time‑based dead‑reckoning (less accurate) + more frequent wall/tape corrections.

- No IMU? Use straight‑line correction by comparing left/right encoders only.

- No docking charger? Still implement return to tape and stop near dock marker; charge manually.

## 17) Test Checklist

- Battery reading matches multimeter (±0.1 V)

- Wet sensor flips reliably with damp cloth; dry recovers within 2 s

- Mop down only on wet; blower never runs when mop is down

- Avoidance: stop <20 cm; recover path

- Line follow stable at 0.2–0.3 m/s

- Clap toggle responds to 2 claps, ignores single clap

- Dock contact: LED on charger shows charging

## 18) What to Hand In (Course Demo)

- Short video showing: start via clap → dry vacuum → transition to wet area → mopping → obstacle avoidance → low‑battery simulated → auto‑return and dock.

- Wiring diagram (block + key connections)

- Code (FSM + modules) with comments

- Tuning notes and limitations (what you’d add next: better mapping, IR beacon homing, brush module, better pump control)

### Appendix A — Quick Dry/Wet Decision Table

| Floor state | Wet contacts | Blower | Mop arm | Pump |
| --- | --- | --- | --- | --- |
| Dry | LOW conduction | ON | UP | OFF |
| Damp | MED conduction | OFF | DOWN | Pulsed |
| Wet/puddle | HIGH conduction | OFF | DOWN | OFF (avoid flooding) |

### Appendix B — Corridor Coverage Basics

- Choose strip spacing slightly less than mop/brush width to avoid gaps.

- Re‑zero yaw each 180° turn to limit drift.

- Mark corridor ends (optional tape markers) to detect strip boundaries reliably.

You can adapt pin mappings or modules you already have; the architecture remains the same. If you want, I can turn this into a starter Arduino sketch tailored to your exact pin choices and modules.